# 03 RASMIY BENCHMARK VA ILMIY VALIDATSIYA HISOBOTI

## 1. Metodologik Asoslar

- Simulyatsiya Muhiti: Eclipse SUMO v1.20+ (Microscopic Traffic Simulation).

- Fizika Rejimi: setSpeedMode(31) (Standart mikroskopik xavfsizlik dinamikasi; teleportatsiya va sun'iy tezlashtirishlar yo'q).

- Takrorlanuvchanlik (Reproducibility): BASE_SEED = 20260829, har bir ssenariy uchun seed = BASE_SEED + run_id va SUMO buyrug'iga --seed parametri uzatilgan.

- Taqqoslanuvchi Asos (Baseline): An'anaviy 80 soniyalik Fixed-Time sikli (35s EW, 5s Sariq/Qizil, 35s NS, 5s Sariq/Qizil).

## 2. A/B Qiyosiy Benchmark Natijalari (Fixed-Time vs TrafficMind)

Bir xil stoxastik sharoitda o'tkazilgan 20 ta mustaqil koridor sinovlari natijalari:

| № | Marshrut Turi | Oqim EW (vph) | Oqim NS (vph) | Fixed-Time Vaqt | Traffic Mind Vaqt | Fixed Kutish | TM Kutish | Yaxshilanish (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | EW_full (Magistral) | 858 | 759 | 172.0s | 111.0s | 87.0s | 25.0s | +35.47% |
| 2 | EW_full (Magistral) | 1003 | 771 | 156.0s | 135.0s | 81.0s | 50.0s | +13.46% |
| 3 | NS_C (Yon ko'cha) | 881 | 735 | 62.0s | 26.0s | 30.0s | 0.0s | +58.06% |
| 4 | EW_full (Magistral) | 774 | 931 | 155.0s | 81.0s | 82.0s | 6.0s | +47.74% |
| 5 | NS_B (Yon ko'cha) | 749 | 811 | 44.0s | 36.0s | 11.0s | 3.0s | +18.18% |
| 6 | NS_D (Yon ko'cha) | 1033 | 768 | 49.0s | 30.0s | 14.0s | 0.0s | +38.78% |
| 7 | EW_full (Magistral) | 716 | 916 | 121.0s | 82.0s | 52.0s | 10.0s | +32.23% |
| 8 | EW_full (Magistral) | 864 | 852 | 129.0s | 93.0s | 56.0s | 15.0s | +27.91% |
| 9 | COMPLEX_TURN (Burilish) | 769 | 772 | 60.0s | 66.0s | 0.0s | 3.0s | -10.00% |
| 10 | EW_full (Magistral) | 724 | 733 | 137.0s | 73.0s | 66.0s | 9.0s | +46.72% |
| 11 | NS_C (Yon ko'cha) | 883 | 981 | 76.0s | 61.0s | 37.0s | 20.0s | +19.74% |
| 12 | COMPLEX_TURN (Burilish) | 1009 | 843 | 73.0s | 65.0s | 10.0s | 3.0s | +10.96% |
| 13 | NS_D (Yon ko'cha) | 912 | 946 | 96.0s | 58.0s | 58.0s | 20.0s | +39.58% |
| 14 | EW_full (Magistral) | 749 | 806 | 113.0s | 87.0s | 43.0s | 10.0s | +23.01% |
| 15 | NS_D (Yon ko'cha) | 807 | 999 | 103.0s | 48.0s | 67.0s | 5.0s | +53.40% |
| 16 | COMPLEX_TURN (Burilish) | 733 | 946 | 116.0s | 71.0s | 53.0s | 9.0s | +38.79% |
| 17 | COMPLEX_TURN (Burilish) | 893 | 900 | 147.0s | 72.0s | 76.0s | 6.0s | +51.02% |
| 18 | EW_full (Magistral) | 1050 | 739 | 163.0s | 148.0s | 83.0s | 57.0s | +9.20% |
| 19 | EW_full (Magistral) | 955 | 783 | 174.0s | 129.0s | 95.0s | 37.0s | +25.86% |
| 20 | NS_D (Yon ko'cha) | 916 | 886 | 85.0s | 51.0s | 46.0s | 12.0s | +40.00% |

### A/B Benchmark Xulosaviy Statistikasi:

- O'rtacha Sayohat Vaqti: Fixed-Time: 111.55s -> TrafficMind: 76.15s (-31.73% tezlashish).

- O'rtacha To'xtab Qolish: Fixed-Time: 52.35s -> TrafficMind: 15.00s (-71.35% qisqarish).

- O'rtacha Samaradorlik O'sishi: +31.51%.

20 scenario ichidan: 19 ta scenario’da TrafficMind V5 travel time bo‘yicha Fixed-Time’dan yaxshi natija ko‘rsatgan. [ 19/20=95% ]

- Natija: 95% test natijalari ozroq sayohat vaqtini ko’rsatdi.

## 3. 120 talik Deterministik Stress-Test Tahlili

120 ta ekstremal ssenariy bo'yicha qat'iy muhandislik qabul mezonlari (Kutish <= 4s va True Delay <= 15s):

- Phase & Preemption Timing (1–30): O'rtacha kechikish 48.72s, to'xtash 13.67s.

- Recovery & Dynamic Gap-Out (31–60): O'rtacha kechikish 19.98s, to'xtash 6.43s, o'tish darajasi 46.7% (14/30).

- Sensor Degradation (61–90): Sensor ishonchliligi Ci = 1.0 dan 0.0 gacha tushganda ham 0 ta qotib qolish (fail-safe ishladi).

- Heavy Congestion & Complex Routes (91–120): Tugallangan 29 ta ssenariy bo‘yicha o‘rtacha kechikish 48.74s, 1 ta timeout (999s) bilan birga hisoblanganda 80.42s, to‘xtash 18.38s.

- Formal Safety Invariant: Barcha 120 ta ssenariyda faza to'qnashuvi soni = 0 ta qoidabuzarlik.

120 ta ssenariyning 119 ta tugallangani bo‘yicha o‘rtacha kechikish 42.47s ni tashkil etdi (1 ta timeout 999s bilan hisoblanganda 50.44s).

## 4. 2,000 talik Stoxastik Monte Carlo Sinovi Tahlili

- Jami Sinovlar (Runs): 2,000 ta tasodifiy koridor simulyatsiyasi (1.1 milliondan ortiq mikroskopik qadam).

- O'rtacha Haqiqiy Kechikish (True Mean Delay): 28.32 soniya (Tugallangan 1,685 ta ssenariy bo‘yicha; 315 ta timeout qat'iy failure sifatida hisoblangan).

- Formal Xavfsizlik Invarianti: 2,000 ta stoxastik run davomida 0 ta faza to'qnashuvi.

- Qat'iy Mezon Bo'yicha O'tish: 469 ta (23.45%) — loyihada belgilangan qat'iy muhandislik mezoni (Kutish <= 4s va Kechikish <= 15s) bo'yicha to'liq yuqori tezlikli o'tishlar soni; qolgan ssenariylar tizimning operatsion chegaralarini aniqlab bergan.

| Metrika Toifasi | Ko‘rsatkich | Qiymat (N=2,000) | Izoh / Ahamiyati | | :--- | :--- | :--- | :--- | | **Reproducibility** | **Base Seed / Policy** | 20260829 / BASE + run_id | Deterministic stoxastik takrorlanuvchanlik | | **Physics & Safety** | **Conflicting Greens** | 0 | Safety Governor formal nazorati ostida | | **Physics & Safety** | **Teleports / Collisions**| 0 | setSpeedMode(31) toza mikroskopik dinamika | | **Functional Tail** | **Mean Travel Time** | 76.15 s | Umumiy o‘rtacha koridor vaqti | | **Functional Tail** | **Mean True Delay** | 28.32 s | Erkin oqimga nisbatan yo‘qotilgan vaqt (T_travel - T_free) | | **Functional Tail** | **Median Delay (P50)** | 25.30 s | Ssenariylarning 50%ida kechikish ushbu qiymatdan past | | **Functional Tail** | **P90 Delay** | 50.34 s | 90% holatlarda kechikish chegarasi | | **Functional Tail** | **P95 Delay** | 59.83 s | Ekstremal oqimlardagi chegaraviy kechikish | | **Functional Tail** | **P99 Delay** | 78.58 s | 1% eng og‘ir tirbandlikdagi kechikish | | **Functional Tail** | **Max Delay/Wait** | 93.84 s | Eng og‘ir ssenariy holati | | **Strict Acceptance** | **Pass Rate** | 469/2,000 (23.45%) | Qat’iy mezon (Wait <= 4s va Delay <= 15s) | I

| Metrika Toifasi | Ko‘rsatkich | Qiymat (N=2,000) | Izoh / Ahamiyati |
| --- | --- | --- | --- |
| Reproducibility | Base Seed / Policy | 20260829/ BASE + run_id | Deterministic stoxastik takrorlanuvchanlik |
| Physics & Safety | Conflicting Greens | 0 | Safety Governor formal nazorati ostida |
| Physics & Safety | Teleports / Collisions | 0 | setSpeedMode(31) toza mikroskopik dinamika |
| Functional Tail | Mean Travel Time | 76.15 s | Umumiy o‘rtacha koridor vaqti |
| Functional Tail | Mean True Delay | 28.32 s | Erkin oqimga nisbatan yo‘qotilgan vaqt (T_travel - T_free) |
| Functional Tail | Median Delay (P50) | 25.30 s | Ssenariylarning 50%ida kechikish ushbu qiymatdan past |
| Functional Tail | P90 Delay | 50.34 s | 90% holatlarda kechikish chegarasi |
| Functional Tail | P95 Delay | 59.83 s | Ekstremal oqimlardagi chegaraviy kechikish |
| Functional Tail | P99 Delay | 78.58 s | 1% eng og‘ir tirbandlikdagi kechikish |
| Functional Tail | Max Delay/Wait | 93.84s | Eng og‘ir ssenariy holati |
| Strict Acceptance | Pass Rate | 469/2,000 (23.45%) | Qat’iy mezon (Wait ≤ 4s va Delay ≤ 15s) |

Izoh: O‘rtacha 28.319 soniyalik kechikish koridordan to‘liq o‘tib ulgurgan 1,685 ta (84.25%) ssenariy bo‘yicha hisoblangan. Qolgan 315 ta (15.75%) ekstremal tirbandlik holatida simulyatsiya vaqti tugab, ularga 999s belgilangan va bu holatlar qat’iy mezon bo‘yicha to‘liq muvaffaqiyatsiz (failed) deb hisobga olingan.